Model:  ARC-57VA

Key Features

|  2MP Full HD Resolution  H.265 Video Compression  45m IR Night Vision  3.6mm Fixed Lens  Built-in Microphone  IP67 Weatherproof & Metal Body |  |
| --- | --- |

Camera Specifications

| Video |  |
| --- | --- |
| Imaging Device | 1/2.7" |
| Effective Pixels | 2MP 1920(H)×1080(V) |
| Scanning System | Progressive CMOS |
| Min. Illumination | Color 0.006lux @ F1.6(AGC ON) ; B/W 0 lux @ IR ON |
| Lens Type |  |
| Focal Length (Zoom Ratio) | 3.6mm |
| Max. Aperture Ratio | F No. 2.2 |
| Angular Field of View | Horizontal: 86°, Vertical: 47°, Diagonal: 105° |
| Min. Object Distance | 2m |
| Lens Type | Fixed(M12) |
| Operational |  |
| IR Viewable Length | Up to 45m |
| Camera Title | On/Off |
| Day & Night | IR cut filter with auto switch（Day/Night/Auto/Image/Schedule） |
| Backlight Compensation | Off/On (5 Zone) |
| Wide Dynamic Range | DWDR |
| Digital Noise Reduction | 3D DNR |
| Motion Detection | Off/On (8 Level) |
| Privacy Masking | Off/On (4 Zone, Rectangle) |
| Gain Control | Auto |
| White Balance | Auto/Manual |
| Electronic Shutter Speed | 1/5 ~ 1/20000s |
| On-Board Storage | Micro SD slot, up to 256GB |
| Alarm | NA |
| Audio | Built-in microphone |
| Hardware Reset | Yes |
| Smart Feature | NA |
| Network |  |
| Ethernet | RJ45(10/100BASE-T) |
| Video Compression Format | H.265/H.264 |
| Resolution | Main Stream &30fps 1080P(1920x1080) Sub Steam &20fps VGA(640x480) |
| Video Quality Adjustment | 256Kbps ~ 4Mbps |
| Bitrate control method | CBR/VBR |
| Streaming Capability | Dual-Stream(Main/Sub) |
| Protocol | TCP/IP,HTTP,DHCP,DNS,DDNS,RTP/RTSP, SMTP,NTP,UPnP,SNMP,HTTPS,FTP |
| Web browser | IE10/11 |
| Security | complicated password;Authenticated username and password |
| Streaming Method | Unicast/Multicast |
| Application Programming Interface | ONVIF |
| Environmental |  |
| Operating Conditions | -35～+60°C/less than ≤ 90% RH |
| Ingress Protection | IP67 |
| Electrical |  |
| Input Voltage / Current | D12V/POE(IEEE802.3af) |
| Power Consumption（IR OFF) | 2.5W-3.5W |
| Power Consumption（IR ON） | 6.5W-8.5W |
| Mechanical |  |
| Material | Metal |
| Dimensions (HxWxD) | 242*87.4*83.88mm |
| Weight | 811g |

Dimension

Accessory